GMT 4.1.4

Orthographic Perspective Projection

-Jg<lon0>/<lat0>/<scale(1:xxxx)>

-Jg<lon0>/<lat0>/<radius>/<lat>

-JG<lon0>/<lat0>/<mapwidth>

lon0/lat0 is the center or the projection.

Scale is either <1:xxxx> or <radius>/<lat>, where <radius> is distance

in [gmtdefs.measure_unit] to the oblique parallel <lat>.

General (Tilted/Vertical) Perspective Projection

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<scale:xxxx>

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<scale:xxxx>

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<width>/<height>/<scale:xxxx>

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<radius>/<lat>

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<radius>/<lat>

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<width>/<radius>/<lat>

-Jg[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<width>/<height>/<radius>/<lat>

-Jg[d|D]       <  1   >/< 2  >/<       3    >/<      4           >/<    5     >/<     6  >/<   7    >/<   8     >/<    9    >/<10> - 9 slashes

-JG[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<mapwidth> - 5 slashes

-JG[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<mapwidth> - 6 slashes

-JG[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<width>/<mapwidth> - 7 slashes

-JG[d|D][s|e]<lon0>/<lat0>/<altitude>/<azimuth[l|L]>/<tilt[l|L]>/<twist>/<width>/<height>/<mapwidth> - 8 slashes

[d|D] – debug switch, “d” turns it on, “D” is very verbose

[s|e] – Earth model, “s” – use spherical, “e” – use given earth elliptical model

Lon0/lat0 is the geodetic location of the point of perspective of the projection.

Altitude is the height of the point of perspective above local geodetic sea level

altitude is in Kilometers

if altitude less than 10 then it is the distance from center of earth to the point of perspective divided by the equatorial radius of the Earth

if altitude is < 0 then altitude is given by the geosynchronous altitude, i.e.~ 37,xxx km, of the point of perspective

Azimuth is degrees east of North of view

Tilt is the upward tilt of the plane of projection

if tilt < 0 then boresight is centered on the horizon

[l|L] is a long/lat switch. With either or both of these switches are present rather than azimuth and tilt these 			values are the boresight long-lat and the azimuth and tilt are computed

Twist is the rotation of field of view clockwise in degrees around boresight

Width is width of the boresight in degrees

If  Height is not supplied the Height is assumed to be the same as the width.

Height is the height of the boresight in degrees

Scale is either <1:xxxx>

or <radius>/<lat>, where <radius> is distance in [gmtdefs.measure_unit] to the oblique parallel <lat>

Snyder, John. P., “Map Projections- A Working Manual”,  US Geological Survey Professional Paper 1395,  (1987) pp 169-181